Stereognosis: Experimental SOP Start

- If recording images.

- If recording neural data.

- Turn on PC1.  
Go to page http://205.208.63.221/mainWK.html#/app/loads and turn on loads 2 (PC2), 3 (PC3) and 8 (Rig).

- Turn on Ephys PC.

- Turn on Nan Drive.

  - Open Nan software.

  - If it shows Communication Error, try changing connection port to any other available. If this doesn’t help or other errors appear, restart PC and try again.

  - When Nan software is on and working, reset all axes to zero. If it prompts you to do it on start, agree.

- Set up OpenEphys:

  - Make sure trigger bnc cable is connected to OpenEphys bnc board.

  - Makes sure OpenEphys is on and indicator lights are green. If not, check that it’s plugged in correctly.

  - Connect all headstages.

  - Open OpenEphys software and load channel configuration depending on the number and type of headstages you connected.

  - Open LFP viewer and Spike viewer in separate windows. Do not press play before this step, it will crash the software.

- Turn on Cameras on PC2 and PC3:

  - Use the CAMERAS link in the middle of the desktop.

  - Press Init Cameras. If the message says that some cameras were not turned on as USB3:

    - Press Release Cameras.

    - Unplug the cameras that were indicated (you are given their cord’s color marking).

    - Plug them back into the SAME port.

    - Go to step 5b.

  - Press Start Server.

- Turn on the Pressure Sensor on PC1:

  - Use the PRESSURE SENSOR link in the middle of the desktop.

  - Press Initialize. Wait – initialization process can take a minute.

  - Press Start Server.

- Generate a new session:

  - Use the folder link SESSION CONFIG on the desktop right above the MAIN PROTOCOL link.

  - In that folder, usually, just double-click the “generate_session.bat”, which should spawn a new session file.

  - Close the folder.

- Start the Main Protocol on PC1:

  - Use the MAIN PROTOCOL link in the middle of the desktop. It can take a few seconds to load.

  - Home (calibrate) the motors:

    - Move the homing metal block into position closest to the middle and screw it in.

    - Press “Home (Calibrate) the motors” and wait for the program to go through each axis. The button will become available when the homing completes.

    - Translation_Z should move the object away from the subject.

    - Tilt should hit the homing metal block.

    - Aperture should move to the widest position. If it has not, run homing again.

    - Move the homing block away from the center to prevent it from interfering with tilt.

  - Connect to 3 servers: 2 camera servers and a pressure sensor server.

  - If the light sensors are not working properly, uncheck the “Use Light Sensors” checkbox.

  - Press Start Protocol.

- Monkey prep in the rig:

  - Put a protective cover around monkey’s head, make sure no cables are within monkey’s reach and nothing is hanging around.

  - Place Nan tower on the chamber.

  - Connect all headstages to probes.

  - Connect grounding cables to each probe. Ground them to the chair.

  - Connect all drives to Nan.

  - Lower guide tubes.

  - Press play in OpenEphys and turn on sound for the lowest channel on the probe (make sure you did everything in 4 first).

  - Start lowering the probes (probably sequentially) and watch for possible bending.

  - When you see/hear first neural activity on a given probe, reset depth for this probe to zero in the corresponding Nan field. Make a note of the reset depth. Do this for all other probes. Adjust spike thresholds if necessary.

  - Lower and map until you are in a location with good units. Make a note of depth and receptive fields of all evident units.

  - Unplug all drives for recording period to avoid extra noise.

- Press Start Trial in the Main Protocol.

  - After the first trial, make sure images, sensor data and neural data are being recorded.

TIPS:

- Steps 3, 4, 5, 6, 7 can be done in any order.

- If the SYNERGY is not working, open it on PC1 and Start/Stop it. Check if all clients connect to it.

Stereognosis: Experimental SOP FINISH

- Deal with the brain first:

  - Makes sure all drives are plugged back in. Raise all probes to zero. Set position to ‘reset depth’ for a given probe and raise to zero again. If you forgot the reset depth, eyeball current drive position and calculate travel distance to ~3mm from the top (better to underestimate than overestimate travel distance).

  - Stop OpenEphys.

  - Unplug all drives, all headstages and all grounding cables.

  - Carefully and slowly raise guide tubes.

  - Unscrew the Nan tower and set it aside.

  - Watch for excessive bleeding. If it happens, before you do anything else, put sterile gauze in and around the chamber. Flush chamber with cold saline until bleeding stops. Clean and close the chamber.

  - Carefully remove probes and guide tubes from the Nan tower, re-attached the probes for cleaning at the edge of the tower. Soak guide tubes and probe tips in Tergazyme solution for half an hour. Then flush with alcohol.

- Close the Cameras Servers on PC2 and PC3.

- Close the Pressure Sensor and Main Protocol on PC1.

- Turn off the power to the RIG using http://205.208.63.221/mainWK.html#/app/loads

- Upload recorded images from PC2 and PC3:

  - Use the folder link “camera_data” underneath the CAMERAS link in the middle of the desktop.

  - Select a folder with today’s date and move its contents into the data folder on the server, e.g.

\\BENSMAIA-LAB\LabSharing\Stereognosis\Data\Daiquiri\2021-02-08\cameras

  - This folder should contain a list of camera directories from the session.

  - Wait for the files to upload (or come back later)

  - Turn off the PC so that cameras do not overheat overnight.

- Upload the pressure sensor data from PC1:

  - Use the folder link “pressure data” underneath the PRESSURE SENSOR link in the middle of the desktop.

  - Select a folder with today’s date and move its contents into the data folder on the server, right next to the cameras, e.g.

\\BENSMAIA-LAB\LabSharing\Stereognosis\Data\Daiquiri\2021-02-08\sensors

- Upload the logs from PC1:

  - Use the folder link “logs” underneath the MAIN PROTOCOL link in the middle of the desktop.

  - Select a CSV and a TXT file with today’s date and move it into the data folder on the server, e.g.

\\BENSMAIA-LAB\LabSharing\Stereognosis\Data\Daiquiri\2021-02-08\

- Upload the neural data (separate folders for continuous and spikes + config file) to the server into

\\BENSMAIA-LAB\LabSharing\Stereognosis\Data\Daiquiri\2021-02-08\

- Close OpenEphys and Nan software. Turn off Nan box, turn off Ephys PC. Unplug headstages from Ephys box and store them separately if a safe clean place.