Standard Operating Procedure for Stereognosis Project

Setup:

- Turn on the power for the rig computers and turn on the computers.

- Turn on power for the motors.

- Turn on the lights.

- Check if cameras were detected properly on each computer:

  - Open SpinView, 4 cameras should be initialized and showing their serials.

  - Click on a camera in the list.

  - Underneath the list with cameras will be the list with parameters, look for AcquisitionFrameRate, it should go up to 199.87 (or close). If it is at lower value (~38), it needs to be reinitialized:

    - Find the camera with that serial number

    - Identify the USB cord that goes from that camera to the PC (it should have the same color tape marking).

    - Unplug it from the PC, wait 10 seconds, plug it back in.

    - Check if the reinitialized camera has the correct high framerate, if not go to step (iii).

- Start Camera Servers on both PCs:

  - Start the App (found in a folder via a “LATEST camera server App” link on desktop).

  - Click Initialize System

  - Choose the correct server setup from Presets (PC1 or PC2)

  - Start Server

- Start Main Protocol on PC1:

  - Start the App (found in a folder via a “LATEST main protocol App” link on desktop).

  - Click Connect to both Camera servers.

  - Set the desired FRAMERATE, GAIN, EXPOSURE and maximum recording period (“Period (s)”.

  - Test connection by Send Config – it should update the relevant parameters on the Camera Servers.

  - Start Protocol

- Start I-Scan on PC2 and prepare to record. During experiment record with 200 Hz for short bursts of time (<10 mins).

- After putting the monkey in place, record a test trial to test the quality of images and lighting. Adjust the lighting and gain to have the optimal illumination of monkey hands. Recorded images can be found in the folder within the “Output Folder” on each Camera Server (preset to be “D:/camera_data/experiment_<DATE_TIME_STAMP>/”.

Record data.

If a camera becomes disconnected due to a USB fault:

- Step 4 from Setup will need to be redone to check if it reinitialized correctly. If the camera keeps disconnecting, try using a different USB port.

- Camera Server will need to be restarted, and camera initialization cycled (Init System, Release System, Init System) and connection to it from the Main Protocol reestablished (disconnect-connect).

The data should be uploaded onto the server (\\BENSMAIA-LAB\LabSharing\Stereognosis\Data\experiment_<DATE>). Use separate folders for PC1 and PC2.